EEC 277

Digital Signal Processing

Final Project Report

AUTHORS

Andrew Sameh Adel Mikhail (G3 S1) – ID: 9489

Mazen Elkaslan (G2 S1) – ID: 9445

Galal Mohamed Taman (G3 S1) – ID: 9453

Throughout this report, the two recommended samples have been used. This includes Record 100 and Record 106 from PhysioNet database.

Part 1:

[1] Problem Definitions and Specifications

| Fig 1.1: Record 100 ECG Signal with normal sinus rhythm | This signal shows a stable periodic rhythm. You can identify a small bump (P-wave) before the larger spike (QRS Complex) and the smaller bump after it (T-wave) as well. |
| --- | --- |
| The flat segments in between each spike are misaligned from the 0mV axis, this is called the Baseline Wonder. It is a relatively clean signal, but one that has slight noise visible in the trace signal. |  |

| This signal shows a similar rhythm, but it also features PVCs (Premature Ventricular Contractions). These are the wider taller peaks after the larger spikes in each period. |  |
| --- | --- |
|  | Fig 1.2: Record 106 ECG Signal with abnormal ventricular ectopic beats |
| If looked closely, there is more visible thickness within the flat segments that suggests a higher level of power-line interference. That is indicated by the subtle oscillations as you zoom in in the flat segments. |  |

Considering these characteristics, the filtering objectives that need to be completed are (1) adjusting the baseline wonder back to the 0mV axis, (2) removing the power-line interference within the abnormal signal and (3) filtering any noise within T- and P- waves and the flat segments.

This will be achieved using 3 filters cascaded in series after one another:

- High-Pass Filter: To remove baseline wander. It allows everything above 0.5Hz to pass.

- Notch Filter: Tuned at 50Hz to remove the power-line interference while leaving surrounding frequencies untouched.

- Low-Pass Filter: To remove the muscle noise and smoothen the signal.

Filter Specifications

When implementing filters, one must consider its properties to attune the filter’s effect to get the desired results. Here are 4 properties for the filters used set in the recommended constraints along with a reason why it was set as such:

|  | Condition | Justification |
| --- | --- | --- |
| Passband Ripple | Very low ripple | If passband ripple is high, it can distort height of the R-peaks or the depth of the S-waves. Can lead to misdiagnosis. |
| Stopband Attenuation | 40dB< As <60dB | Reduce 50Hz power-line noise to 1% of its original amplitude, to make flat segments sharper. |
| Transition Bandwidth | Narrow | Remove exactly the 50Hz without deleting the 48Hz or 52Hz components of QRS complex. |
| Cutoff Frequencies | HPF = 0.5Hz, LPF = 100Hz, Notch = 50Hz | Remove respiration artifacts. Preserve sharp peaks of QRS complex. Standard. |

Definitions:

- Passband Ripple (Rp): The maximum allowable fluctuation in the frequencies we want to keep.

- Stopband Attenuation (As): How much the unwanted noise is weakened.

- Transition Bandwidth (∆f): The speed at which the filter switches from passing to stopping.

Realistic Implementation Constraints

There are a few constraints that need to be considered when these filters are implemented in realistic situations. Here are 4 constraints as suggested in the requirements:

| Constraint |  |
| --- | --- |
| Maximum Filter Order | More memory and processing power is needed, the higher the order of the filter is. We generally use lower orders. |
| Computational Complexity | Every multiplication in the filter consumes battery life. |
| Latency | The time it takes for a heartbeat to go through the filter and appear on a screen. |
| Phase Distortion | Most IIR filters shift different frequencies by different amounts of time, therefore we could use Linear Phase FIR filter to ensure the waves stay where they belong in time. |

[2] Filter Design and Analysis

For the 3 filter types mentioned above, each filter will be assigned a specific digital filter to carry out its role. Details about each digital filter are listed below:

- High-Pass Filter:

- Uses IIR Butterworth, because it has a maximally flat passband.

| Sampling frequency (Fs) | 360Hz |
| --- | --- |
| Cutoff frequency (Fc) | 0.5Hz |
| Filter order | 4 |

Fig 2.1: Magnitude and Phase Response for Butterworth

Fig 2.2: Impulse Response for Butterworth 	Fig 2.3: Step Response for Butterworth

Fig 2.4: Pole-zero diagram for Butterworth

MATLAB Code:

- Notch Filter:

- Uses Chebyshev Type II, because it allows for a flat passband while placing the unneeded noise in the 50Hz band that is filtered out.

| Sampling frequency (Fs) | 360Hz |
| --- | --- |
| Notch frequency (Fn) | 50Hz |
| Bandwidth | 2 |

Fig 2.5: Magnitude and Phase Response for Chebyshev Type II

Fig 2.6: Impulse Response for Chebyshev Type II	Fig 2.7: Step Response for Chebyshev Type II

Fig 2.8: Pole-zero diagram for Chebyshev Type II

MATLAB Code:

\

- Low Pass Filter:

- Uses FIR Window-based filter, because it provides linear phase which ensures that all waves are delayed equally to prevent smearing the QRS complex.

| Sampling frequency (Fs) | 360Hz |
| --- | --- |
| Cutoff frequency (Fc) | 100Hz |
| Stopband Attenuation | 50dB |

Fig 2.9: Magnitude and Phase Response for FIR Window-based

Fig 2.10: Impulse Response for FIR Window-based	Fig 2.11: Step Response for FIR Window-based

Fig 2.12: Pole-zero diagram for FIR Window-based

MATLAB Code:

[3] Implementation and Validation