EEC 277

Digital Signal Processing

Final Project Report

AUTHORS

Andrew Sameh Adel Mikhail (G3 S1) – ID: 9489

Mazen Elkaslan (G2 S1) – ID: 9445

Galal Mohamed Taman (G3 S1) – ID: 9453

Throughout this report, the two recommended samples have been used. This includes Record 100 and Record 106 from PhysioNet database.

Part 1:

[1] Problem Definitions and Specifications

| Fig 1.1: Record 100 ECG Signal with normal sinus rhythm | This signal shows a stable periodic rhythm. You can identify a small bump (P-wave) before the larger spike (QRS Complex) and the smaller bump after it (T-wave) as well. |
| --- | --- |
| The flat segments in between each spike are misaligned from the 0mV axis, this is called the Baseline Wonder. It is a relatively clean signal, but one that has slight noise visible in the trace signal. |  |

| This signal shows a similar rhythm, but it also features PVCs (Premature Ventricular Contractions). These are the wider taller peaks after the larger spikes in each period. |  |
| --- | --- |
|  | Fig 1.2: Record 106 ECG Signal with abnormal ventricular ectopic beats |
| If looked closely, there is more visible thickness within the flat segments that suggests a higher level of power-line interference. That is indicated by the subtle oscillations as you zoom in in the flat segments. |  |

Considering these characteristics, the filtering objectives that need to be completed are (1) adjusting the baseline wonder back to the 0mV axis, (2) removing the power-line interference within the abnormal signal and (3) filtering any noise within T- and P- waves and the flat segments.

This will be achieved using 3 filters cascaded in series after one another:

- High-Pass Filter: To remove baseline wander. It allows everything above 0.5Hz to pass.

- Notch Filter: Tuned at 50Hz to remove the power-line interference while leaving surrounding frequencies untouched.

- Low-Pass Filter: To remove the muscle noise and smoothen the signal.

Filter Specifications

When implementing filters, one must consider its properties to attune the filter’s effect to get the desired results. Here are 4 properties for the filters used set in the recommended constraints along with a reason why it was set as such:

|  | Condition | Justification |
| --- | --- | --- |
| Passband Ripple | Very low ripple | If passband ripple is high, it can distort height of the R-peaks or the depth of the S-waves. Can lead to misdiagnosis. |
| Stopband Attenuation | 40dB< As <60dB | Reduce 50Hz power-line noise to 1% of its original amplitude, to make flat segments sharper. |
| Transition Bandwidth | Narrow | Remove exactly the 50Hz without deleting the 48Hz or 52Hz components of QRS complex. |
| Cutoff Frequencies | HPF = 0.5Hz, LPF = 100Hz, Notch = 50Hz | Remove respiration artifacts. Preserve sharp peaks of QRS complex. Standard. |

Definitions:

- Passband Ripple (Rp): The maximum allowable fluctuation in the frequencies we want to keep.

- Stopband Attenuation (As): How much the unwanted noise is weakened.

- Transition Bandwidth (∆f): The speed at which the filter switches from passing to stopping.

Realistic Implementation Constraints

There are a few constraints that need to be considered when these filters are implemented in realistic situations. Here are 4 constraints as suggested in the requirements:

| Constraint |  |
| --- | --- |
| Maximum Filter Order | More memory and processing power is needed, the higher the order of the filter is. We generally use lower orders. |
| Computational Complexity | Every multiplication in the filter consumes battery life. |
| Latency | The time it takes for a heartbeat to go through the filter and appear on a screen. |
| Phase Distortion | Most IIR filters shift different frequencies by different amounts of time, therefore we could use Linear Phase FIR filter to ensure the waves stay where they belong in time. |

[2] Filter Design and Analysis

For the 3 filter types mentioned above, each filter will be assigned a specific digital filter to carry out its role. Details about each digital filter are listed below:

- High-Pass Filter:

- Uses IIR Butterworth, because it has a maximally flat passband.

| Sampling frequency (Fs) | 360Hz |
| --- | --- |
| Cutoff frequency (Fc) | 0.5Hz |
| Filter order | 4 |

Fig 2.1: Magnitude and Phase Response for Butterworth

Fig 2.2: Impulse Response for Butterworth 	Fig 2.3: Step Response for Butterworth

Fig 2.4: Pole-zero diagram for Butterworth

MATLAB Code:

- Notch Filter:

- Uses Chebyshev Type II, because it allows for a flat passband while placing the unneeded noise in the 50Hz band that is filtered out.

| Sampling frequency (Fs) | 360Hz |
| --- | --- |
| Notch frequency (Fn) | 50Hz |
| Bandwidth | 2 |

Fig 2.5: Magnitude and Phase Response for Chebyshev Type II

Fig 2.6: Impulse Response for Chebyshev Type II	Fig 2.7: Step Response for Chebyshev Type II

Fig 2.8: Pole-zero diagram for Chebyshev Type II

MATLAB Code:

\

- Low Pass Filter:

- Uses FIR Window-based filter, because it provides linear phase which ensures that all waves are delayed equally to prevent smearing the QRS complex.

| Sampling frequency (Fs) | 360Hz |
| --- | --- |
| Cutoff frequency (Fc) | 100Hz |
| Stopband Attenuation | 50dB |

Fig 2.9: Magnitude and Phase Response for FIR Window-based

Fig 2.10: Impulse Response for FIR Window-based	Fig 2.11: Step Response for FIR Window-based

Fig 2.12: Pole-zero diagram for FIR Window-based

MATLAB Code:

[3] Implementation and Validation

Now that all the required filters have been designed and implemented individually, they will be combined to accomplish the stated objectives above.

Here is the MATLAB code used to define all the filters in one program, pass the signals of both Record 100 and Record 106 through the cascaded filters, and display a few comparison plots to prove the filters effects and the output signals:

NOTE: Consider ecg100 and ecg106 as signal data matrixes for Record 100 and Record 106 respectively, and t as follows in the variable assignment.

Now ecg100_final and ecg106_final include the filtered output signals for both Record 100 and Record 106 respectively.

The continuation of the MATLAB code below begins plotting all comparison plots and graphs for analysis and validation.

Fig 3.1: Before and after filtering signal plots

Fig 3.2: SNR improvement print output

Fig 3.3: Spectrogram of both output signals

Fig 3.4: Power Spectral Density of both output signals

As shown in Figure 3.1, the signal baseline for both filtered signals is now centered at 0mV. This is proof that the low frequency DC offset was successfully removed by the high-pass filter.

Noticing the deep line dive that occurs at exactly 50Hz in both signals in the power spectral density diagrams, that concludes that the power interference we discussed above has been successfully removed by the notch filter.

Now you might notice that in the filtered signals, the noise within the flat segments has not been removed completely. That might lead to the question of whether the signal never had noise as we thought it did or that the filter never did work, but there is proof that the low-pass filter did actually work.

Look at Figure 3.3, the bright yellow vertical lines, which represent the heartbeats, are still bright and sharp. This means the signal was never distorted, and the areas between 100Hz and 180Hz have signs of noise removal.

Figure 3.4 shows that unwanted frequencies (<0.5 Hz, 50 Hz, and >100 Hz) are strongly reduced in the filtered signals.

The SNR improvement calculation goes a step further to prove that the quality of the output signal has improved compared to the raw one, without sacrificing QRS complex integrity.

Distortions and Possible Limitations

- FIR low‑pass filter: Works well to remove muscle noise, but needs many coefficients (higher order), which makes it slower and heavier for real‑time systems.

- IIR filters: More efficient (lower order), but can cause phase distortion if not handled carefully.

- Residual noise: Some muscle noise overlaps with useful ECG frequencies, so removing it completely could also weaken real signal details.

Part 2:

This is the MATLAB program code for the multi-band digital equalizer for speech clarity enhancement for audio files.